# Health Monitoring System for Elderly

Problem Statement: Elderly people often live alone and lack immediate access to health monitoring and emergency alerts.

Objective: Create a system that monitors pulse rate, temperature, and fall detection, and sends alerts if abnormalities are detected.

Requirements: NodeMCU ESP8266 Pulse Sensor DHT11 Sensor MPU6050 Accelerometer (for fall detection) Blynk or ThingSpeak

Expected Outcome: A compact wearable system that sends health status and emergency alerts to caregivers or medical professionals.

MY EXPERIENCE

As part of the RISE internship, I created a simulated health monitoring system for the elderly. This project combined multiple sensors and conditions to detect potential health emergencies like high temperature, abnormal pulse rate, and fall detection.

Since I didn’t have a pulse sensor, I creatively used a potentiometer connected to analog pin A0 to simulate heartbeats. I mapped its output to realistic BPM values (40–130) and set alert thresholds.

For temperature, I used my existing DHT22 sensor connected to pin A2, and for fall detection, I used an MPU6050 accelerometer over I2C (SDA → A4, SCL → A5). If the system detected high fever, irregular pulse, or sudden motion (g-force > 2.0), it would trigger a buzzer and LED alert.

This project pushed me to understand how multiple inputs can work together to make a real-life safety solution. I also learned how to simulate hardware creatively using analog input when a sensor is unavailable — a skill that helped me stay on track without giving up.

Most importantly, this gave me confidence in handling multi-sensor logic, simulation testing, and translating ideas into meaningful real-time outputs.